Proposer:

Nick Roads

Seconder:

Caryl Wills

Proposal

To include a UK Hybrid class in the 2016 technical regulations.

Purpose

To allow for European Endurance racing a UK Hybrid car to have a clearly defined set of regulations. 

This class is only for European racing and it is NOT proposed to be allowed to compete in UK race series or championship.

Class differences from Standard or Economy class:

- 2 cylinder BMW R series 1100 RS as a replacement for 602 cc engine. Wiring to be free but engine to use unmodified ECU which comes with bike.

- Dyane/ 2CV gearbox (8:31 differential). Bell housing modifications to allow for 10 mm shortening.

- We do not see that mounting is required under the technical regulations but if these are required can provide drawings. We are using 3 main mounts/ plates which fit within and on standard chassis.

- Modified (from BWM original) inlet manifold so that the throttle body can easily mount air filters

Probably the 2CVRT one:

- 2CVRT air restrictor on the throttle body to keep the power reasonable for the car

Probably the 2CVRT supplied restrictors – definitely same dimensions:

- A class specific exhaust system from BMW engine to run to left hand side of car and then free to any exhaust as for 2CV. We can provide detailed photos for these if required.

- A new spacer from BMW original and gearbox adapter plate. 2cv flywheel as per UK regs would need different location holes.

- Any fuel system to provide fuel injection but this must work with standard 2cv tank. Typically 2 fuel pumps, swirl pot and it requires a return line to tank.

- We may need to have a modified braking system – discs, calipers and pads.